Blink scores (blendshapes)

| Column | Description |
| --- | --- |
| eyeBlinkLeft | Left eye blink score (0–1) |
| eyeBlinkRight | Right eye blink score (0–1) |
| eyeWideLeft | Left eye wide-open score |
| eyeWideRight | Right eye wide-open score |
| eyeSquintLeft | Left eye squint score |
| eyeSquintRight | Right eye squint score |

Blink asymmetry

| blink_lr_asym | eyeBlinkLeft − eyeBlinkRight (+ve = left more closed) |
| --- | --- |

Nasal/temporal eyelid aperture asymmetry

| LE_aperture_temporal | Left eye aperture at temporal third (normalised by eye width) |
| --- | --- |
| LE_aperture_central | Left eye aperture at centre |
| LE_aperture_nasal | Left eye aperture at nasal third |
| LE_nt_asym | Left eye nasal − temporal aperture (+ve = nasal-wider) |
| RE_aperture_temporal | Right eye aperture at temporal third |
| RE_aperture_central | Right eye aperture at centre |
| RE_aperture_nasal | Right eye aperture at nasal third |
| RE_nt_asym | Right eye nasal − temporal aperture |

Head pose & distance

| head_yaw | Head rotation left/right (degrees) +ve right turn |
| --- | --- |
| head_pitch | Head tilt up/down (degrees) + chin up |
| head_roll | Head tilt side-to-side (degrees) + tilt right |
| distance_cm | Estimated camera-to-face distance (cm, from Z-translation) |

EAR (Eye Aspect Ratio — geometric openness)

| left_ear | Left eye geometric aperture |
| --- | --- |
| right_ear | Right eye geometric aperture |